[image: image1.emf][image: image2.png]9“5|N7~€
Q&

‘A[L

(oA
'VVuv‘\

’(733‘0



NIT       


CCNP 1 - BSCI

 re Skills Exam

Version 3, 2008

Duration: 3 hours
including 30 minutes of design

[image: image3.png]HOGSKOLAN VAST



Requirements

1. Configure according to the diagram shown above.

2. Add the loopback interfaces on R2 into OSPF area 20. 

3. Summarize the loopback interfaces on R2 to the backbone with the most efficient mask

4. Redistribute R1’s loopback 0 interface into OSPF.

5. Configure R3 to distribute, by OSPF, a default route to 192.168.43.4 .
     

6. Configure R3 to be in BGP AS 65100 and R4 to be in BGP AS 65400 .

7. Peer R3 and R4 in BGP on both WAN links.

8. Advertise R4’s loopback interfaces into BGP.

9. Make BGP on R3 advertise the 172.16.0.0/16 network.

10. Make AS 65100 prefer the T3 link to AS 65400 networks by modifying the localpreference on R3.

11. Make AS 65400 prefer the T3 link to AS 65100 networks by modifying the MED on R3.
     

12. Configure IPv6 and OSPFv3 on R1 and R2, including the Lo1 on R1.
(If you can't configure IPv6: your score will be maximum 70, but you will not fail)

Checkpoints

A) On R1: ping 172.31.12.1 source 172.16.1.1 

B) On R4: ping 172.16.1.1 source 172.31.12.1 

C) No explicit static routes, no redistribution (except Lo0 on R1 into OSPF) but implicit rules like “ip route 0.0.0.0 ????  ????“  and  “ip route  ????  ????  null 0“ are OK

D) On R2: ping 2001:6b0:2d1:1::1

